Storage Server

S640-G20

S640-G20 storage server is a high-performance two-way high-density storage server based on Intel latest Grantley platform developed by Sugon. Processor adopt new Haswell architecture, compared with last platform, and it has more cores, cache and memory capacity, and larger, faster communication channel, which can help to rapid transmission of data. Supporting maximum 1.5 TB memory, integrated high-performance SAS controller, supporting maximum 36 pieces of hot plug 3.5 inch SAS/SATA hard disk, the strong extensibility can support the operation of the key tasks, meet the needs of memory-intensive applications. S640 -- the G20 server is very applicable to finance, security, transportation, post, telecommunication, energy and internet etc., which are some industry data center and remote enterprise environment with stringent demands on the server performance storage capacity, scalability, and reliability. Overview. New Processor Architecture Used the Intel's latest generation E5-2600 v3 series processors, providing higher performance for the user application. Intel Xeon E5-2600V3 series processors Intel Xeon E5-2600 compared with the previous generation of products, it has various major improvements, including supporting more cores, upgrading and supporting DDR4 memory. Due to more and faster QPI bus adopted between processer, Improved collaboration between CPU efficiency drastically. Powerful Processing Performance Processor integrated 4 channels memory controller ， and support maximum 2133 MHZ DDR4 memory, memory bandwidth performance can be increased 35%. Used Intel new C610 series chipset, supporting SATA, SATA RAID expansion, providing for user more choices. Rich extensibility Each CPU with 12 root memory slot, support DDR4 memory, memory capacity can be extended to 1.5 TB, provide a flexible and powerful memory configuration options System support maximum 36 hard disk slots, and provide the possibility for user structure the mass storage. Eight I/O expansion slot with PCI - E 3.0 slot, realize the flexible expansion, to satisfy different customer's high performance, high reliability, and low latency requirements. High Density Adopted 4U rack type design, chassis in high strength and high stiffness material, unique design of server heat dissipation structure in high-density rack type, whit high performance, high density and high reliability..

Features:

- Haswell architecture

- High density architecture, Sufficient storage capacity

- Ultra strong  I/O expansibility，leading industry with Ultra High performance

- True enterprise

- Reliability and availability

- Convenient function of deploy and management

Product features

Functional features

Advantages

High redundancy design technology guarantees the reliability and availability of the system effectively, to ensure the system no single point of failure

System configuration with redundant power supply and fan module，which support hot plug，to ensure the high reliability of the system，is very suitable for running business-critical applications；Effective thermal design ensures the reliability of the system，extends the service life of components effectively, and reduces the cost.

New processor architecture, improve application performance significantly

Adopted new Intel Xeon E5-2600 V3 series processor, many major technological improvement, improved the performance of the system greatly；integrated PCI - E controller in processor，realize lower latency and power consumption，and higher total capacity and broadband ;PCI - E 3.0 bus provide high speed channel support for high-performance card devices；adopted more and faster QPI bus between the processor，Improve collaboration efficiency between CPU.

A variety of energy-saving technology, greatly reduce the cost of system energy consumption and heat dissipation

Powerful scalability and advanced flexibility, can meet the needs of changing business

Provide the mass storage space, can meet the demand of memory-intensive applications for performance and capacity

System can be configured maximum 36 pcs hard disk,，compatible SATA/SAS/SSD and other different types of hard disk，single disk capacity maximum 6 TB， can provide 216 TB of mass storage space，with the board integrated 12 gb SAS interface and high performance RAID card，to  provide senior RAID protection function for user data, and achieve high availability of system，to meet the demand of memory-intensive applications for performance and capacity.

Rich management functions, convenient for management and maintenance of the system

System integration BMC intelligent control chip and IPMI management front-end ports，provide hardware levels of access and control functions, Support remote control, management, deployment operating system and application system，provide convenient management ability for large-scale system deployment and remote distributed applications，can roundly monitoring operation state of system， realize the separation of business and management，improve management efficiency, and reduce management costs.

System provides 24 DIMM slot，memory maximum1.5TB，Such high extensibility is very suitable for compute-intensive applications and virtualization environment；Providing as many as eight new generation high speed PCIe 3.0 expansion slots, which can realize two times the PCIe 2.0 bandwidth with lower latency,，provide users with high I/O expansion ability，and support a variety of high-performance outer card， meet the needs of high-end customers on the system function and performance.

Adopted the Intel new architecture processor，with more sensors, finer control, faster control circuit and higher precision， compared with the previous generation of products has larger ascension in terms of energy consumption；adopted high-effective power supply module and intelligent speed control fan，and efficient heat dissipation design，can reduce the unnecessary power losses， significantly reduce the overall power consumption of the system， can meet the special needs of users for power consumption.

Technical specifications

| Host Performance |  |  |  |
| --- | --- | --- | --- |
| Processor | Supporting Intel Xeon E5-2600V3 series of multi-core processors |  |  |
| Internal Memory | 24 root DDR4 memory slot, extended maximum 1.5 TB |  |  |
| Network Controller | Integrated two-port GBPS or two-port XFP |  |  |
| PCI I/O Expansion Slot | Supporting maximum 8 PCI-E expansion slot |  |  |
| Hard Disk Controller | The integration eight SAS controller can be selected，support RAID 0、1 The integration eight SAS RAID controller，support RAID 0、1、5、6 SAS or SAS RAID cards are optional |  |  |
| Storage | Supporting maximum 36pcs hot plug 3.5/2.5 inch SAS/SATA hard disk，Supporting SAS/SATA hard disk mixed interpolation，and at the same time supporting maximum 4 pcs rear hot plug 2.5 -inch SATA/SAS hard disk或SSD hard disk |  |  |
| Power Supply | 1 + 1 redundant power supplies |  |  |
| Management Function | Integrated BMC chip，supporting IPMI2.0 and KVM Over IP senior management function The special front pane with hard disk status on regional instructions can be selected |  |  |
| Video Card | Integrated display controllers, 16 MB video memory |  |  |
| Supporting Operating Systems | Supporting for mainstream Windows、Red hat Enterprise Linux、SUSE Linux Enterprise Server，the details OS version please see the list of compatible products |  |  |
| Case | 4U rack-mountable server case |  |  |
| Case Dimensions | 886mm（D）×432mm（W）×176mm(H) |  |  |
| Weight | Maximum 70 kg |  |  |
| The requirements of environmental temperature | Working:10℃～35℃(50℉～95℉) Storing: -40℃～55℃(-40℉～131℉) |  |  |
| Relative humidity requirements | Working maximum relative humidity 90% RH (40 ℃) Working 35％～80％ RH Transportation and storage 20％～93％ RH |  |  |
| Shake | frequency 5Hz～150Hz, acceleration≤20m/s2 , amplitude≤0.15mm |  |  |
| Impact peak acceleration 150m/s2～300m/s2, duration≤11ms |  |  |  |
| Collision | peak acceleration 100m/s2～150m/s2,times less than1000 |  |  |
| For more information: Please visit official Sugon website: www. Sugon.com or dial the customer service hot line: 400-810-0466 You can also contact Sugon branches distributed throughout the state. Please log in the official website of Sugon for contact information. Sugon reserves the right to modify the product specifications or other product information (including, but not limited to weights, appearance, dimensions or other physical elements) without any further notification. Any changes of information involved in the document caused by products upgrading or other causes, no additional notification will be provided. All product graphics involved in the document shall be subject to the physical products. |  |  |  |